| Yaesu VX-7R USA | TX 5W@50,144,430 1.9A; 0.3W@222 1.0A | RX 0.5-999,0.2A Mono; 0.24A Dual | 07/31/2026 |
| --- | --- | --- | --- |

| Band | AM B’cast | Shortwave | FM B’cast | VHF-TV | Ham 50 | Ham 144 | Ham 222 | Ham 430 | AIR | Action 1 | Action 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RX MAIN | 0.5-1.8 | 1.8-30 | 59-108 | 174-222 | 30-58.99 | 137-173.995 | 222-224.98 | 420-469.975 | 108-136.975 | 225-419.9875 | 803-823.9875 849-998.9875 |
| RX SUB |  |  |  |  | 30-58.99 | 137-173.995 |  | 420-469.975 |  |  |  |
| TXMARS/CAP |  |  |  |  | 50-54 | 140-173.995 | 222-224.98 | 420-469.975 |  |  |  |
| TXfreeband |  | 40-224.95 |  |  | 30-58.99 | 137-173.995 | 222-224.98 | 420-469.975 | 108-136.975 | 300-579.995 |  |

| Press | Alternate function | “1” | “2” | “3” | Band | Upper (MAIN) TX |
| --- | --- | --- | --- | --- | --- | --- |
| Long | VFO: Memory write MR: set Priority channel | Store HP1 | Store HP2 | Store HP3 | Band | Mono Band <> Dual Band |
|  | No action (two presses turns on backlite with null effect) | Scan. PTT to cancel. | Dual Watch | Weather |  Band | Mono Band Only VFO: Upper Font Size (Mono only) MR: Freq <> Freq & Alpha Tag |
| Press | Misc #2 RV: Simplex on rptr TX HM: Band Home channel | “4” | “5” | “6” | “0” | Lower (SUB) TX |
| Long | Activate EMERGENCY | Store HP4 | Store HP5 | Store HP6 | Store HP0 | Mono Band <> Dual Band |
|  | Recall Home Channel for band | ARTS on/off | Smart Search | Spectrum Analyzer | Enter Set mode or DIAL, PTT to set squelch | Mono Band Only VFO: Lower Font Size (Mono only) MR: Freq <> Freq & Alpha Tag |
|  |  |  |  |  |  | MONI |
| Press | Internet Connection | “7” | “8” | “9” | VFO <> Memory | Open Squelch |
| Long | Key Lock (2 sec.) | Store HP7 | Store HP8 | Store HP9 | VFO: No action MR: PMS Scan | Open Squelch |
|  | TX power (band ¦ ch) L1(0.05) > L2(1.0) > L3(2.5) > high(5.0) | Ch counter (determine frequency) | CTCSS/DCS | Special Memory mode | No action | Set Squelch level |

| Hyper Memory - | To store current radio config, - 2 seconds. To recall, ; ; DIAL to #4 HYP; PTT; - . To activate a different HYP, stop scanning on primary band (PTT); - . To exit HYP mode, ; ; DIAL to #1 OFF; PTT. |
| --- | --- |
| DTMF Manual | While pressing PTT, enter DTMF string on MIC keypad. (TSQ #7 must be OFF & must be #1 OFF.) |
| DTMF Autodialer | Turn on DTMF dialer (TSQ #7). Press & hold PTT. Number - corresponding to desired DTMF string. Release PTT. |
| Memory store | In VFO mode: set RX freq (DIAL or keypad), CTCSS vs. DCS (TSQ #1,2,3,6), repeater offset (Basic #6,7), power level (), Mode (Basic #4), scan mode (Basic #13), & display alpha tag vs. frequency (). ½ sec. Within 5 seconds, to store Home Channel for band or select channel (DIAL; watch for  for available) and to store. (If Ln/Un does not appear, switch to MR mode & turn of Memory Bank mode.) Set SCAN MODE (Basic #13) if desired. |
| Memory recall | In MR mode, DIAL or enter channel number using keypad and . |
| Alpha tag (max 8 char.) | Recall memory channel. Select Basic #11. . DIAL clockwise to move to next character. Press key repeatedly for alphanumerics (: ABCabc2) or for spec characters ()*+-/=. When done, to complete entry. PTT to save setting. During entry, to delete all characters to the right of cursor. |
| Mask/unmask memory channel | In MR mode, ½ second. DIAL to desired channel number. . |
| Delete memory ch | Mask it. |
| Memory Group (1-9) Note: Memory Groups only work on MAIN. However, concurrent MAIN MG scan and SUB Pref Mem Scan are possible. | To assign channel to Memory Group, recall memory channel; ½ second; number - corresponding to desired MG. To activate a MG, (if not already on MAIN); ; ; DIAL to #2 MR GROUP; PTT; DIAL to select MG1 - MG9; . To activate a different MG, until “MG” blinks; DIAL; . To exit MG mode, . ; DIAL to #1 OFF; PTT. To delete a channel from a MG, enter MG mode; select channel; for 2 seconds; . |
| Odd split | For splits not available via Basic #6, see Operating Manual p46. If using VX-7 Commander, set RPT SH = “Dup” and Shift = tx freq. |
| Xfer Memory to VFO | Recall memory channel. ½ second. . |
| MR only mode (no VFO possible) | Press and hold while turning radio on. To return to normal operation, repeat. |
| One Touch Memory | To store desired frequency/attributes into channel OTM0 - OTM9, see Memory Store. To recall, in Mono Band mode on MAIN, ; ; DIAL to #3 OTM; PTT; - . To exit OTM mode, ; ; DIAL to #1 OFF; PTT. |
| Weather | DIAL, preferably on SUB. To scan, PTT. To stop scan, PTT. To manually select channel, DIAL. To exit WX, . For WX Alert, set WX Alert (Misc #20) ON, then initiate WX scan (PTT). Scan will watch for 1050Hz tone on any WX channel. Normal radio operations can be performed on other band. (SUB is preferable for WX Alert, because MAIN has more features, e.g., Memory Groups.) |
| Squelch | . . DIAL. PTT. |
| Pref Mem Scan | In MR mode, select any channel with . . . |
| Skip memory - temp | While stopped on the channel during scan, . . User experiences vary as to whether Basic #13 is set to SKIP temporarily or permanently. Whether this function works at all may depend up whether you are scanning all or scanning a Memory Group. |
| PMS (Band Limit Scan) | Program lower limit & upper limit (L1/U1 - L20/U20) as described in “Memory Store”. Remember tuning step (Basic #3). In MR mode, select Ln. ½ sec to set PMS mode. “PMS” appears. . .to initiate scan. To stop scan, PTT. |
| Dual Watch | In MR mode, ½ second. Select priority memory channel. . “MRP” (MAIN) or “MRp” (SUB) appears. Pick any other channel or select VFO mode, . . Priority memory channel is checked every 5 sec. to cancel. |
| Spectrum Analyzer | Set Spectrum Analyzer mode (Scan #6) to 1 or CONTINUOUS. In VFO Mono Band mode, activate ().To adjust bandwidth, or . To change center freq, DIAL. to operate on center (and displayed) channel. |
| Smart Search (15-1-15 channels) | Set Smart Search mode (Scan #5) to 1 or CONTINUOUS. In VFO Mono Band mode, select center frequency. Adjust squelch. To activate mode (). ”SMRT” appears. To start/stop search,. To review channels, DIAL. To exit, . |
| Channel Counter (identify freq.) | Set sweep width (Scan #1). In VFO Mono Band mode, activate (). 50dB attenuator. To manually stop, . |
| Tone scan | VFO or MR: Set TSQ #1. Enter edit mode for TSQ #2 (CTCSS) or TSQ #3 (DCS). to enable editing. to search for tone/code. Once found, to select value. PTT to save setting. During scanning, MONI, while pressed, opens audio. |
| Reset | Microprocessor: Hold while turning on radio. .Menu settings: Hold while turning on radio. . |

. DIAL to select item # (or , DIAL to cycle to item #1 of each menu group, then , DIAL to select item #). ( is sometimes required to enable editing.) or to select value. PTT to save setting.

| Basic Setup | 1 |  | m |  | Squelch NFM: 0 - 15 1 | Measurement | 1 | r |  |  | Sensor display: TIME DC TEMP WAVE BARO ALTI WX OFF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 | r |  |  | Squelch WFM: 0 - 8 2 |  | 2 | r |  |  | Wave Form monitor: ALL RX_SIGNAL TX_MODULATION |
|  | 3 |  | m | c | VFO Step: 5 9 10 12.5 15 20 25 50 100kHz |  | 3 | r |  |  | Temp measurement units: °C/°F |
|  | 4 |  | m | c | mode: Auto NFM AM WFM |  | 4 | r |  |  | Barometer units: hpa mbar mmHg inch |
|  | 5 | r |  |  | ARS(Auto Repeater Shift): On Off |  | 5 | r |  |  | Barometer offset |
|  | 6 |  |  | c | Repeater Shift: 0.00 - 99.95MHz |  | 6 | r |  |  | Altimeter units: m ft |
|  | 7 |  |  | c | Repeater Shift Direction: -RPT +RPT SIMP |  | 7 | r |  |  | Altimeter offset |
|  | 8 | r |  |  | Audio Mute: On Off | Save Modes | 1 | r |  |  | Auto Power Off (APO) OFF 30min 1hour 3hour 5hour 8hour |
|  | 9 | r |  |  | Key Beep: On Off |  | 2 | r |  |  | RX battery sleep ratio: OFF 2001:1 3001:1.5 5001:2.5 11:5 21:10 |
|  | 10 | r |  |  | Lockout: KEY DIAL KEY&DIAL PTT KEY&PTT DIAL&PTT ALL |  | 3 | r |  |  | TX battery save (lowers TX power if strong RX): ON OFF |
|  | 11 |  |  | c | Alpha tag |  | 4 | r |  |  | TOT (Time Out Timer): OFF 1_min 2.5_min 5_min 10_min |
|  | 12 | r |  |  | Memory write mode: LOWER CH (next avail) NEXT_CH (end) |  | 5 | r |  |  | Radio ON time of day: OFF 00:00 - 23:59 |
|  | 13 |  |  | c | Memory scan mode: OFF SKIP PREFERENTIAL |  | 6 | r |  |  | Radio OFF time of day: OFF 00:00 - 23:59 |
|  | 14 | r |  |  | Hyper Memory write: ENABLE DISABLE | ARTS | 1 | r |  |  | Beep: IN RANGE ALWAYS OFF |
| Display | 1 | r |  |  | RX Busy LED - squelch open: ON OFF |  | 2 | r |  |  | Polling interval: 15 SEC 25_SEC |
|  | 2 | r |  |  | Contrast: 1 - 10 7 |  | 3 | r |  |  | CW ID |
|  | 3 | r |  |  | Brightness: 0 - 12 10 | Miscellaneous | 1 | r |  |  | Busy Channel Lock-Out (no TX if RX w/o TSQ/DCS): ON OFF |
|  | 4 | r |  |  | When power off: None TEMP ALL |  | 2 | r |  |  | Function of HM/RV : HOME REV |
|  | 5 | r |  |  | Lamp mode: KEY(5 sec.) CONTINUE OFF |  | 3 | r |  |  | Function of MONI button: MONI T-CAL(1750Hz for Europe) |
|  | 6 | r |  |  | LED Color 1 RGB |  | 4 | r |  |  | Exchange MonF and MONI : FUNC MONI |
|  | 7 | r |  |  | LED Color 2 for MainBusy SubBusy Dual MainTX SubTx CHG |  | 5 | r |  |  | “Emergency” alarm: BEEP+STROBE BEEP STROBEn BEAM |
|  | 8 |  | m |  | Meter Symbol: S1 S2 S3 S4 S5 CHR |  | 6 |  |  | c | Reduce TX bandwidth by 50% (to 2.5kHz): ON OFF |
| TSQ/DCS/DTMF | 1 |  | m | c | Type: OFF TONEd TONE_SQLe/d DCSe/d [if #6 is ON: D_CODEe TONEe_DCd DCe_TONEd] |  | 7 | r |  |  | VOX Sensitivity: OFF HIGH LOW |
|  | 2 |  | m | c | Tone value: 50 CTCSS values 88.5 |  | 8 | r |  |  | VOX delay: 0.5S 1S 2S |
|  | 3 |  | m | c | DCS code: 104 DCS codes 023 |  | 9 | r |  |  | Band Link: ON OFF |
|  | 4 | r |  |  | DCS invert: ENABLE DISABLE |  | 10 |  | b |  | VFO Band Edge for current band: ALL BAND |
|  | 5 | r |  |  | RX CTCSS/DCS bell ringer repetitions: OFF 1 3 4 8 CONTINUE |  | 11 | r |  |  | Font editor |
|  | 6 | r |  |  | Split CTCSS/DCS coding: OFF ON |  | 12 | r |  |  | Icon display: ON OFF |
|  | 7 | r |  |  | DTMF Dialer: ON OFF |  | 13 | r |  |  | Icon editor |
|  | 8 | r |  |  | DTMF program |  | 14 | r |  |  | Icon selection |
| Scan | 1 | r |  |  | Channel counter search width in MHz: ±5 ±10 ±50 ±100 |  | 15 |  |  | c | Clock shift: ON OFF |
|  | 2 | r |  |  | Band-edge beep: ON OFF |  | 16 | r |  |  | Set time of day. Press V/M button before PTT. |
|  | 3 | r |  |  | Scan resume: 3SEC 5SEC 7SEC 10SEC BUSY HOLD |  | 17 | r |  |  | Language: ENGLISH JAPANESE |
|  | 4 | r |  |  | Scan lamp: ON OFF |  | 18 | r |  |  | Front end 20dB attenuator: ON OFF |
|  | 5 | r |  |  | Smart Search Sweep mode: 1 CONTINUOUS |  | 19 | r |  |  | Mic monitor (VX7 mic  TX & external earphone): ON OFF |
|  | 6 | r |  |  | Spectrum Analyzer Sweep mode: 1 CONTINUOUS |  | 20 | r |  |  | WX alert: ON OFF |
|  | 7 | r |  |  | Spectrum Analyzer audio of center frequency: ON OFF |  |  |  |  |  | Coding: radio main/sub band channel |

| Special Memory: ; DIAL to select, PTT. To exit, ; DIAL #1 Off, PTT. 1; Off; Normal keypad (VFO freq; MR channel). 2; MG; Memory Group; MAIN only. 3; OTM; One Touch Memory; Mono Band & MAIN. 4; HYP; Hyper Memory. 5; LST; Shortwave Broadcast*; Mono Band & MAIN. 6; SEA; VHF Marine*; Mono Band & MAIN. *Channel cannot be selected via direct keyboard entry; only via DIAL. Memory Group: MG; Channels; PREFERENTIAL. 1; Ham; 1-49; 146.85 (Cody), 146.52(Call2m), 446.000(Call70cm). 2; Cody L.E. & Fire; 50-99. 3; Cody BLM, Natl Forest; 168.55(BLM2IC), 168.625(Air***). 4; NIRSC; 140-199. 5; FRS/GMRS/MURS; 310-336. 6. 7; Aircraft. 8; Yellowstone N.P.; 136-139. 9; Search & Rescue (SAR); 250-269; 155.16(SAR***;SAR national). Hyper Memories: HYP; Top; Bottom; MAIN/SUB. 0; Cody 2m repeater (146.850); (Mono Band); MAIN. 1; Scan Ham (MG1); Scan Pref(MG all); SUB. 2; Scan L.E. (MG2); Scan Pref(MG all); SUB. 3; Scan BLM, Natl Forest (MG3); Scan Pref(MG all); SUB. 4; Scan NIRSC (MG4); Scan Pref(MG all); SUB. 5. 6; Cody 2m repeater (146.850); Shell repeater (146.610); MAIN. 7; Cody 2m repeater (146.850); Call 2m (146.520); MAIN. 8; Scan Yellowstone N.P. (MG8); Scan Pref(MG all); SUB. 9; Scan SAR (MG9); Scan Pref(MG all); SUB. |  |
| --- | --- |